# Criticality Safety Evaluation: Cascade Cylinder Array

## Safe-by-Design Demonstration

| Document Information |  |
| --- | --- |
| Document Number | CSE-2026-001 |
| Revision | 0 |
| Date | 2026-02-13 |
| Status | DRAFT - FOR REVIEW |

| Prepared By | Reviewed By | Approved By |
| --- | --- | --- |
| Date: | Date: | Date: |

## Executive Summary

Finding: The cascade cylinder array is safe by design. Criticality is not credible given operating conditions and equipment design.

### Why UO2F2 Cannot Cause Criticality

|  | Critical Threshold | Max Accumulation | Margin |
| --- | --- | --- | --- |
| Fill fraction | 2.5% | 0.29% | 8.6× |
| Mass per cylinder | 2.5 kg | 283 g | 8.8× |

- The critical threshold (2.5% fill) represents the minimum amount of material needed for criticality

- Maximum accumulation (0.29% fill) assumes worst case: 100% humidity, 100% water deposition, 100% reaction to UO2F2 at peak moderation (H/U=25), over entire 10-year equipment lifetime

- Even under these extreme assumptions, accumulation reaches only 1/9th of the critical threshold

### Why UF6 Cannot Cause Criticality

|  | Critical Threshold | Operating Limit | Margin |
| --- | --- | --- | --- |
| Mass per cylinder | 5.2 kg | 5 g | 1,040× |

- The critical threshold (5.2 kg solid UF6) is 1,000× higher than the operating limit (5 g)

- UF6 would need to solidify and continuously accumulate to reach criticality — this is not credible given process conditions

### Conclusion

No additional criticality controls are required. Safety is ensured by physical limits on material accumulation, not administrative controls.

## Table of Contents

- References

- Purpose

- Inputs

- Modeling Assumptions

- Methodology

- Results

- Accumulation Analysis

- Safety Margin

- Conclusions

Appendices - A: Geometry Validation - B: Density Calculations - C: Accumulation Calculation - D: Complete Results Data

## 1. References

| Reference | Title |
| --- | --- |
| OpenMC | Monte Carlo Particle Transport Code, Version 0.15.x |
| ENDF/B-VIII.0 | Evaluated Nuclear Data File, Release VIII.0 |

## 2. Purpose

This analysis evaluates criticality safety for the cascade cylinder array by determining:

- Critical threshold — What fill fraction causes criticality? (Sections 5–6)

- Maximum accumulation — What is the maximum credible material buildup? (Section 7)

- Safety margin — Is the margin sufficient for safe-by-design? (Section 8)

Three material scenarios representing upset conditions in a cascade are evaluated:

| Scenario | Material | Scenario |
| --- | --- | --- |
| UF6 Dry | Solid UF6 | Direct UF6 is deposited in maker |
| UO2F2 Dry | Dry UO2F2 (H/U=0) | UO2F2 is deposited in maker |
| UO2F2 Wet | Wet UO2F2 (H/U=25) | Bounding case of UO2F2 with peak moderation |

## 3. Inputs

### 3.1 Array Configuration

| Parameter | Value | Basis |
| --- | --- | --- |
| Array | 3 × 4 × 5 | Unit cell (rows × cols × layers) |
| Boundary | Reflective | Models infinite array |
| Enrichment | 21 wt% U-235 | HALEU is 20% so we add additional 1% enrichment |

### 3.2 Geometry

Geometry Cross-Sections

Figure 3.1: Unit cell geometry. Left: XY plane (top view). Right: XZ plane (side view). Note top and bottom caps are not shown because of pixel fidelity but they are there

### 3.3 Cylinder Dimensions

| Parameter | Value | Units |
| --- | --- | --- |
| Inner diameter | 25.4 | cm |
| Internal height | 100.0 | cm |
| Wall thickness | 0.3175 | cm |
| Horizontal gap | 12.7 | cm |
| Vertical gap | 7.62 | cm |
| Internal volume | 50,671 | cm³ |

### 3.4 Material Properties

| Material | Density (g/cc) |
| --- | --- |
| UF6 (solid) | 5.09 |
| UO2F2 (dry) | 6.37 |
| UO2F2 (H/U=25) | 1.95 |

Note: UF6, UO2F2 (dry) densities reported from Wikipedia. UO2F2 wet density calculated from ideal mixing (see Appendix B).

## 4. Modeling Assumptions

We setup an infinite array of casettes and make several conservative modeling assumptions to assess criticality.

### 4.1 Conservative Assumptions

| # | Assumption | Justification |
| --- | --- | --- |
| 1 | Infinite array (reflective BC) | Bounds any finite array configuration |
| 2 | 21 wt% enrichment | Maximum credible HALEU for this process |
| 4 | H/U = 25 for wet UO2F2 | Peak moderation ratio (Section 6.2) |
| 5 | All water ingress reacts to UO2F2 and deposits in machine | Bounds water ingress and UO2F2 formation |
| 6 | 10-year equipment lifetime | Full design life |
| 7 | All machines accumulate uniformly | In practice, only failed seals leak and accumulate fissile material significantly. |

## 5. Methodology

### 5.1 Approach

- Run Monte Carlo simulations sweeping fill fraction (α) for each material

- Identify critical threshold: fill fraction where k-eff + 2σ ≥ 0.95

- Convert threshold to mass (kg)

- Compare to maximum credible accumulation

- Calculate safety margin

### 5.2 Monte Carlo Parameters

| Parameter | Value |
| --- | --- |
| Code | OpenMC v0.15.x |
| Nuclear data | ENDF/B-VIII.0 |
| Particles per batch | 10,000 |
| Total batches | 150 (50 inactive) |

Safety Classification: - SAFE: k-eff + 2σ < 0.95 - MARGINAL: 0.95 ≤ k-eff + 2σ < 1.00 - CRITICAL: k-eff + 2σ ≥ 1.00

### 5.3 Fill Fraction (α)

Fill fraction α is the fraction of cylinder volume occupied by fissile material:

α=VfissileVcylinder

At fill fraction α, the fissile mass per cylinder is:

m=α×Vcylinder×ρ

## 6. Results

### 6.1 k-effective vs Fill Fraction

#### UF6 Dry

| fill (%) | k-eff + 2σ | Status |
| --- | --- | --- |
| 1% | 0.772 | SAFE |
| 2% | 0.960 | MARGINAL |
| 5% | 1.139 | CRITICAL |
| 10% | 1.220 | CRITICAL |
| 20% | 1.266 | CRITICAL |

#### UO2F2 Dry (H/U = 0)

| fill (%) | k-eff + 2σ | Status |
| --- | --- | --- |
| 1% | 0.885 | SAFE |
| 2% | 1.062 | CRITICAL |
| 5% | 1.223 | CRITICAL |
| 10% | 1.296 | CRITICAL |
| 20% | 1.341 | CRITICAL |

#### UO2F2 Wet (H/U = 25, Bounding)

| fill (%) | k-eff + 2σ | Status |
| --- | --- | --- |
| 2.0% | 0.846 | SAFE |
| 2.5% | 0.953 | MARGINAL |
| 3.0% | 1.037 | CRITICAL |

### 6.2 H/U Ratio Optimization

Peak reactivity occurs at H/U = 25 (k-eff = 1.650 at 100% fill). This ratio is used for the bounding wet UO2F2 analysis.

| H/U | k-eff + 2σ |
| --- | --- |
| 0 | 1.387 |
| 20 | 1.649 |
| 25 | 1.652 |
| 30 | 1.649 |
| 50 | 1.605 |

### 6.3 Combined Results

Combined Fill Sweep

Figure 6.1: k-effective + 2σ vs fill fraction for all three material scenarios. Horizontal lines show safety limit (0.95) and critical (1.00). Vertical line shows maximum credible accumulation (0.29%).

### 6.4 Critical Thresholds

| Material | Critical α | Mass per Cylinder |
| --- | --- | --- |
| UF6 Dry | ~2% | 5.2 kg UF6 |
| UO2F2 Dry | ~1.5% | 4.8 kg UO2F2 |
| UO2F2 Wet (H/U=25) | ~2.5% | 2.5 kg mixture |

## 7. Accumulation Analysis

### 7.1 Mechanism

UO2F2 forms when humid air leaks into UF6-containing equipment:

UF6+2H2O→UO2F2+4HF

### 7.2 Input Parameters

| Parameter | Value | Basis |
| --- | --- | --- |
| Leak rate | 0.01 torr/min | Conservative differential pressure (provided by Riane and Wisher) |
| Humidity | 100% relative humidity | Bounding |
| Temperature | 25°C | Ambient |
| Equipment volume | 12 L | Equipment spec |
| Lifetime | 10 years | Design life |

### 7.3 Result

| Parameter | Value |
| --- | --- |
| UO2F2 accumulated per machine | 163.6 g |
| Water associated (H/U=25) | 119.6 g |
| Total wet mixture mass | 283.2 g |
| Mixture volume (at 1.95 g/cc) | 145.2 cc |
| Fill fraction | 0.29% |

See Appendix C for full calculation.

### 7.4 UF6 Accumulation

Per process engineering, the maximum UF6 holdup at any given time is conservatively 5 g per machine.

| Parameter | Value |
| --- | --- |
| UF6 max per machine | 5 g |
| Fill fraction | 0.002% |

## 8. Safety Margin

| Material | Critical Threshold | Max Accumulation | Margin |
| --- | --- | --- | --- |
| UF6 Dry | 2% (5.2 kg) | 5 g | 1,040× |
| UO2F2 Wet | 2.5% (2.5 kg) | 283 g (0.29%) | 8.6× |

Bounding Case: UO2F2 wet at H/U=25 with 8.6× safety margin. UF6 has an even larger margin (1,040×) but UO2F2 wet is the limiting scenario.

## 9. Conclusions

The cascade cylinder array is safe by design. (See Executive Summary)

| Scenario | Critical Threshold | Max Accumulation | Margin |
| --- | --- | --- | --- |
| UF6 Dry | 5.2 kg | 5 g (operating limit) | 1,040× |
| UO2F2 Wet | 2.5 kg | 283 g (lifetime accumulation) | 8.6× |

Key points: - UO2F2: Even if all water deposits and reacts over 10 years at peak moderation (H/U=25), accumulation is 8.6× below critical - UF6: Operating limit is 1,000× below critical, and would require solidification and continuous accumulation

Recommendations: 1. No additional criticality controls required 2. Maintain existing leak-tightness verification program 3. Obtain chemistry SME validation of material properties (Section 4.2)

## Appendix A: Geometry Validation

Location: _validation/ and summary_plots/

| File | Description |
| --- | --- |
| geometry.png | XY and XZ cross-sections |
| geometry_thick_wall.png | Enhanced view with visible caps |

## Appendix B: Density Calculations

### B.1 UO2F2 Wet Density (Ideal Mixing)

When UO2F2 is hydrated, the mixture density is:

ρmixture=MUO2F2+nH2O×MH2OVUO2F2+nH2O×VH2O

Where: - nH2O=H/U2 (moles water per mole UO2F2) - MUO2F2=308.03 g/mol - MH2O=18.02 g/mol - VUO2F2=308.036.37=48.35 cc/mol - VH2O=18.021.00=18.02 cc/mol

### B.2 Calculated Densities

| H/U | n_H2O | Mass (g) | Volume (cc) | ρ (g/cc) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 308.03 | 48.35 | 6.37 |
| 10 | 5.0 | 398.13 | 138.45 | 2.88 |
| 20 | 10.0 | 488.23 | 228.55 | 2.14 |
| 25 | 12.5 | 533.28 | 273.60 | 1.95 |
| 30 | 15.0 | 578.33 | 318.65 | 1.82 |

### B.3 Mass Conversion

Fissile mass at fill fraction α:

m=α×50,671 cm3×ρ

| Material | ρ (g/cc) | Mass at 1% fill |
| --- | --- | --- |
| UF6 | 5.09 | 2.58 kg |
| UO2F2 dry | 6.37 | 3.23 kg |
| UO2F2 wet (H/U=25) | 1.95 | 0.99 kg |

## Appendix C: Accumulation Calculation

### C.1 Water Ingress Rate

Water mole fraction in saturated air:

fH2O=RH×PsatPatm=1.0×23.8760=0.0313

Water partial pressure rise rate:

dPH2Odt=0.0313×0.01=3.13×10−4 torr/min

### C.2 Lifetime Accumulation

Lifetime in minutes:

t=10×365×24×60=5,256,000 min

Total water pressure:

ΔPH2O=3.13×10−4×5,256,000=1,646 torr=2.17 atm

Moles of water (ideal gas law):

nH2O=2.17×120.0821×298.15=1.06 mol

### C.3 UO2F2 Formation

From stoichiometry (UF6 + 2H2O → UO2F2 + 4HF):

nUO2F2=1.062=0.531 mol

mUO2F2=0.531×308.03=163.6 g

### C.4 Wet Mixture at H/U=25

At peak moderation (H/U=25), each mole of UO2F2 is associated with 12.5 moles of water:

mH2O=0.531×12.5×18.015=119.6 g

Total wet mixture mass:

mmixture=163.6+119.6=283.2 g

### C.5 Fill Fraction

Using wet UO2F2 density at H/U=25 (from ideal mixing, see Section 4.2):

ρH/U=25=1.95 g/cc

Vmixture=283.21.95=145.2 cc

α=145.250,671=0.00287=0.29%

## Appendix D: Complete Results Data

Location: runs/*/latest/results.csv

| Run | Description | Cases |
| --- | --- | --- |
| uf6_dry_low_fill | UF6 at 1-20% fill | 6 |
| uo2f2_dry_low_fill | UO2F2 dry at 1-20% fill | 6 |
| uo2f2_hu_sweep | H/U sweep at 100% fill | 11 |
| uo2f2_wet_peak | UO2F2 H/U=25 at 2-3% fill | 3 |

End of Document