# Criticality Safety Analysis

# HALEU Parallel Pipes Configuration

Calculation Number: [To Be Assigned]

Revision: 0

Date: 2026-02-09

| Role | Name | Date |
| --- | --- | --- |
| Prepared by |  |  |
| Reviewed by |  |  |
| Approved by |  |  |

# 1. References

ANSI/ANS-8.1-2014, "Nuclear Criticality Safety in Operations with Fissionable Materials Outside Reactors"

10 CFR 70.24, "Criticality Accident Requirements"

NUREG/CR-6698, "Guide for Validation of Nuclear Criticality Safety Calculational Methodology"

OpenMC Monte Carlo Code (open-source particle transport)

ENDF/B-VIII.0 Nuclear Data Library

# 2. Purpose

This analysis determines the criticality safety of parallel cascade piping configurations containing high-assay low-enriched uranium (HALEU) in the form of solid UF6. The study evaluates a configuration of three parallel horizontal pipes arranged in a row, as would be found in cascade header piping or process manifolds.

The analysis addresses the following criticality safety questions:

What is the maximum reactivity for 3 parallel pipes at HALEU enrichments (20-24%)?

How does pipe diameter (NPS 3" to 6") affect criticality safety?

What pipe spacing (edge-to-edge gap) is required for adequate subcriticality?

This parametric study sweeps enrichment levels from 20% to 24% (HALEU range), pipe sizes from NPS 3" to NPS 6", and gap distances from 2 cm to 6 cm to map the safety envelope for these configurations.

# 3. Inputs

## 3.1 Geometry Visualization

Figure: Geometry Cross-Section

Figure: XY cross-section showing three parallel pipes with UF6 (yellow), stainless steel walls (gray), and water moderator/reflector (blue).

## 3.2 Pipe Configuration

| Parameter | Value | Units |
| --- | --- | --- |
| Number of pipes | 3 | - |
| Pipe arrangement | Horizontal, parallel | - |
| Pipe orientation | Horizontal (along X-axis) | - |
| Pipe length | 200 | cm |

## 3.3 Pipe Geometry (per NPS)

Standard schedule 40 pipe dimensions:

| NPS | Outer Diameter | Wall Thickness | Inner Diameter |
| --- | --- | --- | --- |
| 3" | 8.89 cm | 0.549 cm | 7.79 cm |
| 4" | 11.43 cm | 0.602 cm | 10.23 cm |
| 5" | 14.13 cm | 0.655 cm | 12.82 cm |
| 6" | 16.83 cm | 0.711 cm | 15.41 cm |

## 3.4 Materials

| Material | Composition | Density (g/cc) |
| --- | --- | --- |
| UF6 | Solid uranium hexafluoride | 5.09 |
| Pipe wall | Stainless steel 304 | 8.0 |
| Moderator/Reflector | Water | 0.5 (worst-case) |

Note: Water at 0.5 g/cc represents worst-case moderation conditions (mist, fog, or partial flooding). This density produces peak reactivity for multi-unit arrays.

## 3.5 Parameter Ranges Analyzed

| Parameter | Values | Units |
| --- | --- | --- |
| Enrichment | 20, 24 | wt% U-235 |
| Pipe size (NPS) | 3, 4, 5, 6 | inches |
| Edge-to-edge gap | 2, 4, 6 | cm |
| Water density | 0.5 | g/cc |
| Water reflector thickness | 30 | cm |

Total cases analyzed: 2 enrichments × 4 pipe sizes × 3 gaps = 24 cases

# 4. Assumptions

## 4.1 Worst-Case Moderation

For multi-unit configurations, the most reactive water density is NOT fully flooded (1.0 g/cc). Instead, intermediate densities (~0.5 g/cc) can produce higher k-eff values due to the balance between:

Moderation benefit: Water thermalizes neutrons, increasing fission probability

Absorption penalty: More water = more neutron absorption between units

A preliminary moderation sweep identified ~0.5 g/cc as peak reactivity, representing conditions such as:

Water mist/fog accumulation

Partial flooding scenarios

Fire suppression spray conditions

## 4.2 Conservative Assumptions Summary

| Assumption | Value Used | Justification |
| --- | --- | --- |
| UF6 form | Pure solid | Bounds actual chemistry (complexes reduce reactivity) |
| UF6 density | 5.09 g/cc | Maximum solid density |
| Fill level | 100% | Assumes fully loaded pipes |
| Temperature | Room temperature | Most reactive condition |
| Enrichment | Up to 24% | Bounds HALEU operations |
| Water density | 0.5 g/cc | Peak reactivity from moderation sweep |
| Reflection | 30 cm water | Full reflection on all sides |
| Pipe wall | SS304 | Realistic for process piping |

# 5. Analytical Methods and Computations

## 5.1 Monte Carlo Code

Code: OpenMC (open-source Monte Carlo particle transport)

Nuclear data: ENDF/B-VIII.0 continuous-energy cross sections

Thermal scattering: S(α,β) treatment for hydrogen in water

## 5.2 Simulation Parameters

| Parameter | Value |
| --- | --- |
| Particles per batch | 10,000 |
| Total batches | 150 |
| Inactive batches | 50 |
| Active batches | 100 |
| Total histories | 1,000,000 |

## 5.3 Geometry Model

3D explicit geometry: Three horizontal cylinders modeled individually

Pipe components: UF6 core, stainless steel wall

Water: Surrounds all pipes and fills gaps between them

Boundary conditions: Vacuum at outer boundaries

## 5.4 Statistical Uncertainty

Expected 1σ uncertainty: ~80-100 pcm

Safety margin: k-eff + 2σ used for all safety determinations

Status classification:

SAFE: k-eff + 2σ < 0.95

MARGINAL: 0.95 ≤ k-eff + 2σ < 1.0

CRITICAL: k-eff + 2σ ≥ 1.0

# 6. Results

## 6.1 Summary Statistics

| Metric | Value |
| --- | --- |
| Total cases analyzed | 24 |
| Cases SAFE | 24 |
| Cases MARGINAL | 0 |
| Cases CRITICAL | 0 |
| k-eff range | 0.4985 - 0.6936 |

## 6.2 k-effective Results

k-eff by Enrichment and Pipe Size (Gap = 2 cm)

| Enrichment | NPS 3" | NPS 4" | NPS 5" | NPS 6" |
| --- | --- | --- | --- | --- |
| 20% | 0.5015 | 0.5690 | 0.6200 | 0.6711 |
| 24% | 0.5158 | 0.5855 | 0.6415 | 0.6922 |

k-eff by Enrichment and Pipe Size (Gap = 4 cm)

| Enrichment | NPS 3" | NPS 4" | NPS 5" | NPS 6" |
| --- | --- | --- | --- | --- |
| 20% | 0.5072 | 0.5756 | 0.6257 | 0.6750 |
| 24% | 0.5207 | 0.5898 | 0.6442 | 0.6936 |

k-eff by Enrichment and Pipe Size (Gap = 6 cm)

| Enrichment | NPS 3" | NPS 4" | NPS 5" | NPS 6" |
| --- | --- | --- | --- | --- |
| 20% | 0.4985 | 0.5643 | 0.6127 | 0.6649 |
| 24% | 0.5126 | 0.5803 | 0.6326 | 0.6834 |

## 6.3 Effect of Pipe Size

Larger pipe sizes result in higher k-effective values due to increased fissile mass:

At 20% enrichment: NPS 3" to NPS 6" increases k-eff by Δk = 0.168

At 24% enrichment: NPS 3" to NPS 6" increases k-eff by Δk = 0.173

## 6.4 Effect of Gap Distance

The effect of edge-to-edge gap on k-effective is minimal for these configurations:

At 20% enrichment, NPS 6": Gap 2 cm to 6 cm changes k-eff by Δk = -0.0062

At 24% enrichment, NPS 6": Gap 2 cm to 6 cm changes k-eff by Δk = -0.0088

The negligible gap sensitivity indicates that pipe spacing has minimal impact on criticality for these far-subcritical configurations.

## 6.5 Visualization

Figure: k-eff vs Pipe Size

Figure: k-eff Heatmap

# 7. Conclusions

## 7.1 Safety Status

All 24 analyzed cases are SAFE (k-eff + 2σ < 0.95).

The maximum k-effective observed was 0.694 (at 24% enrichment, NPS 6", 4 cm gap), which provides a substantial safety margin below the 0.95 acceptance criterion.

## 7.2 Maximum k-eff by Configuration

| Enrichment | Pipe Size | Max k-eff | Safety Margin to 0.95 |
| --- | --- | --- | --- |
| 20% | NPS 6" | 0.6750 | 0.2750 |
| 24% | NPS 6" | 0.6936 | 0.2564 |

## 7.3 Key Findings

Three parallel pipes are inherently safe for HALEU enrichments up to 24% in pipes up to NPS 6", even at worst-case moderation conditions.

No spacing restrictions required: All analyzed gap distances (2-6 cm) are safe. Even touching pipes (gap = 0) would likely remain subcritical given the large safety margins observed.

Pipe diameter is the dominant parameter: Increasing pipe size from NPS 3" to NPS 6" increases k-eff by approximately 0.17, compared to negligible (<0.01) changes from gap distance.

Conservative bounding analysis: These results use worst-case moderation (0.5 g/cc water) and maximum solid UF6 density (5.09 g/cc). Actual operating conditions would produce lower reactivity.

## 7.4 Operational Recommendations

Based on this analysis, the following operational guidance is provided:

No minimum spacing required between parallel pipes up to NPS 6" at enrichments up to 24%

No administrative controls needed for 3-pipe configurations in this size range

Standard pipe installation practices are acceptable from a criticality safety standpoint

# 8. Attachments

Attachment A: Geometry validation plots (XY, XZ cross-sections)

Attachment B: Full results CSV file

Attachment C: Input YAML configuration file

Attachment D: OpenMC input files (sample case)

Report generated by Crit-Buddy